Essay Title:

Predictive Maintenance in Smart Manufacturing: Evaluating AI Models for Equipment Failure Prediction

Programme title:

MSc Artificial Intelligence

Prepared by:

Amaan Mohammed Khalander

Year:

Term 1 – 2026

Submission Date:

17 / June / 2026

Issued by:

Dr. Abdelaziz Triki

Table of Contents

Executive Summary	4

1. Introduction	5

2.  Literature Review (2021–2025)	6

2.1 Dominant Methodologies	6

2.2 Benchmark Datasets	6

2.3 Research Gaps	7

3. AI Model Selection and Justification	8

3.1 Long Short-Term Memory (LSTM) Networks	8

3.2 Temporal Convolutional Networks (TCNs)	8

4. Data Requirements and Preprocessing	10

4.1 Data Requirements	10

4.2 Handling Class Imbalance	10

4.3 Feature Extraction	10

4.4 Data Splitting	10

5. Experimental Design	12

5.1 Evaluation Metrics	12

5.2 Cross-Validation Strategy	12

5.3 Comparison Criteria	12

6. Critical Analysis: Interpretability, Scalability, and Deployment	14

6.1 Interpretability	14

6.2 Scalability	14

6.3 Deployment	14

7. Concluding Remarks	16

References	17

List of Tables

Table 1: Comparative Summary of Selected AI Models	8

Table 2: Recommended Preprocessing Pipeline	11

Table 3: Experimental Design Summary	12

Statement of compliance with academic ethics and the avoidance of plagiarism

I honestly declare that this dissertation is entirely my own work and none of its part has been copied from printed or electronic sources, translated from foreign sources and reproduced from essays of other researchers or students. Wherever I have been based on ideas or other people texts I clearly declare it through the good use of references following academic ethics.

(In the case that is proved that part of the essay does not constitute an original work, but a copy of an already published essay or from another source, the student will be expelled permanently from the postgraduate program).

Name and Surname:

AMAAN MOHAMMED KHALANDER

Date: 27 / May / 2026

# Executive Summary

This essay investigates AI-driven predictive maintenance (PdM) for smart manufacturing, focusing on sensor-based fault detection and Remaining Useful Life (RUL) estimation. Drawing on twelve peer-reviewed sources (2021–2025), it evaluates dominant methodologies, benchmark datasets, and key research gaps.

Two models are compared: Long Short-Term Memory (LSTM) networks and Temporal Convolutional Networks (TCNs). The essay covers data requirements, preprocessing, experimental design, and a critical analysis of interpretability, scalability, and deployment. It concludes that a hybrid TCN–tree-based strategy is the most industrially defensible approach.

# 1. Introduction

Unplanned equipment downtime imposes significant production and repair costs on industrial manufacturers. Reactive and preventive maintenance strategies are increasingly inadequate in Industry 4.0 environments, where interconnected sensor networks generate continuous streams of vibration, temperature, pressure, and acoustic data. Predictive maintenance (PdM) uses AI models to forecast failures before they occur, enabling proactive scheduling and cost reduction (Bampoula et al., 2024).

In the case study, a smart manufacturing company operates pumps, motors, and conveyor systems, each instrumented at high frequency. Management requires a solution to estimate the RUL of each component and flag anomalies ahead of breakdowns. This essay addresses all five brief tasks: literature review (Task 1), model selection (Task 2), data preprocessing (Task 3), experimental design (Task 4), and critical analysis of deployment considerations (Task 5).

# 2.  Literature Review (2021–2025)

AI-driven PdM research has grown substantially since 2021, fuelled by affordable sensors and maturing deep learning toolchains. This section critically synthesises twelve peer-reviewed sources across methodologies, datasets, and gaps.

## 2.1 Dominant Methodologies

Recurrent deep learning dominates recent PdM literature. Bampoula et al. (2024) compare LSTM autoencoders with Transformer encoders on manufacturing time-series data, finding that Transformers better capture long-range temporal dependencies under non-stationary conditions. This extends their 2021 work (Bampoula et al., 2021), which showed LSTM autoencoders detecting anomalies in cyber-physical systems without domain expertise reflecting a broader shift of NLP architectures into industrial sensor analytics.

Temporal Convolutional Networks (TCNs) have gained substantial traction as an alternative. Fatahi et al. (2024) introduce a TCN–Autoencoder with Multi-Head Attention for gas turbine fault detection, outperforming LSTM-Autoencoder and GRU-Autoencoder baselines with MSE 1.447, RMSE 1.193, and MAE 0.712. TCNs use dilated causal convolutions enabling full parallelisation, a scalability advantage over sequential LSTM computation (Abdelhedi and Bouri, 2025).

Ensemble methods represent a third strand. Rezk et al. (2024) combine Deep Reinforcement Learning (DRL), Random Forest (RF), and Gradient Boosting Machines (GBM), demonstrating that hybrid ensembles reduce false-positive alerts in imbalanced IIoT datasets. Separately, Elsherif et al. (2025) propose a CNN-LSTM hybrid (CAELSTM) achieving RMSE 14.44 and 13.40 on NASA C-MAPSS FD001 and FD003, highlighting how piecewise degradation modelling addresses missing ground-truth labels.

## 2.2 Benchmark Datasets

The NASA C-MAPSS dataset has four sub-datasets (FD001–FD004) with 21 sensor signals for turbofan engine degradation remains the dominant RUL benchmark (Sharma, 2024). Its ubiquity enables cross-study comparisons but risks benchmark overfitting; architectural choices tuned for turbofan patterns may not transfer to pumps or conveyor systems. The PRONOSTIA bearing dataset, from the IEEE PHM 2012 Challenge, provides run-to-failure data for rolling element bearings under three load conditions. Pandya et al. (2021) achieve over 90% accuracy on PRONOSTIA using an Autoencoder-LSTM, making it the more relevant primary benchmark for the present manufacturing context. Elkateb et al. (2024) caution that both datasets omit the sensor drift, noise, and irregular sampling typical of live production environments.

## 2.3 Research Gaps

Three gaps are consistently acknowledged. First, labelled failure data are scarce: machines are engineered not to fail, so run-to-failure records are costly and rare, yielding severely imbalanced datasets (Rezk et al., 2024; Elkateb et al., 2024). Second, cross-machine generalisation is poor. Most models are validated on a single equipment type, and domain transfer (e.g., turbofan-to-pump) remains largely unsolved (Bampoula et al., 2024). Third, real-time edge deployment is under-studied; deep learning models are computationally heavy for embedded micro-controllers (Alotaibi and Aly, 2025; Aboshosha et al., 2023). This essay addresses gaps one and three through model choice and the deployment analysis in Section 6.

# 3. AI Model Selection and Justification

LSTM networks and TCNs are selected on the basis of the sensor data characteristics high-frequency, multivariate time series and the dual task requirements of fault classification and RUL regression.

## 3.1 Long Short-Term Memory (LSTM) Networks

LSTMs are a specialised RNN class that overcome the vanishing gradient problem via input, forget, and output gates, controlling information flow across time steps (Bampoula et al., 2021). This makes them well-suited to sequential sensor data where temporal ordering carries diagnostic value such as a rising vibration trend preceding bearing failure. Elsherif et al. (2025) and Pandya et al. (2021) confirm strong LSTM performance on C-MAPSS and PRONOSTIA respectively, establishing it as the most validated PdM baseline. Its key limitation is sequential computation: inference latency and training cost scale unfavourably with window length, and internal gating states offer no intuitive mapping to physical fault mechanisms.

## 3.2 Temporal Convolutional Networks (TCNs)

TCNs replace recurrence with dilated causal convolutions at exponentially increasing dilation rates, capturing long-range dependencies while processing all time steps in parallel (Fatahi et al., 2024). Fatahi et al. (2024) and Abdelhedi and Bouri (2025) demonstrate superior or competitive accuracy versus LSTM variants, with the added benefits of faster training and CNN-native hardware acceleration on edge devices. The principal limitation is a bounded receptive field: capturing very long degradation horizons requires many convolutional layers, increasing memory usage. TCN tooling is also less mature than LSTM across major frameworks.

Table 1: Comparative Summary of Selected AI Models

| Dimension | LSTM | TCN |
| --- | --- | --- |
| Architecture | Gated recurrent units; sequential | Dilated causal convolutions; parallel |
| Temporal range | Short–medium via gating | Long-range via dilation |
| Training speed | Slower (sequential) | Faster (parallelised) |
| Interpretability | Low (opaque gates) | Low (abstract filters) |
| Benchmark (RUL) | RMSE 14.44, C-MAPSS FD001 | MSE 1.447, gas turbine data |
| Edge deployability | Moderate (ONNX) | Good (CNN-native) |

# 4. Data Requirements and Preprocessing

## 4.1 Data Requirements

Each machine class requires multivariate sensor streams: vibration (vertical and horizontal accelerometers), temperature, pressure (piezoelectric), and acoustic emission. Vibration sampling must reach at least 10–25.6 kHz to resolve bearing fault frequencies (Elkateb et al., 2024). Metadata including machine ID, load/speed settings, and maintenance logs are needed for supervised labelling. Where run-to-failure data are unavailable, PRONOSTIA and C-MAPSS serve as validated surrogates.

## 4.2 Handling Class Imbalance

Healthy observations vastly outnumber failure precursors in industrial streams, causing standard cross-entropy loss to default to the majority class. Elkateb et al. (2024) apply Borderline-SMOTE to synthesise minority-class samples in feature space. Cost-sensitive learning and Balanced Random Forests offer complementary strategies by penalising misclassification of the minority class or resampling mini-batches at training time (Rezk et al., 2024).

## 4.3 Feature Extraction

Features are extracted across three domains. Time-domain statistics (RMS, kurtosis, crest factor, peak-to-peak) are sensitive to incipient bearing defects. FFT-derived frequency features expose characteristic fault frequencies such as inner and outer ball-pass rates. Wavelet packet decomposition and STFT provide time-frequency representations suited to variable-speed machinery. For sequence models, a 30-step sliding window with 50% overlap generates training samples; the window length is tuned as a hyperparameter. Elsherif et al. (2025) use piecewise linear degradation labels per window when only total run-to-failure length is available.

## 4.4 Data Splitting

Data are split 70/15/15% (train/validation/test), strictly respecting temporal order to prevent data leakage. Where multiple machines exist, a machine-stratified split holding out entire machines for testing better represents deployment on unseen equipment than random splitting.

Table 2: Recommended Preprocessing Pipeline

| Stage | Action | Rationale |
| --- | --- | --- |
| 1. Acquisition | Vibration, temperature, pressure, acoustic ≥10 kHz | Resolve bearing fault frequencies |
| 2. Cleaning | Impute gaps; remove sensor dropouts | Maintain temporal continuity |
| 3. Normalisation | Min–max or z-score per channel | Equalise feature magnitudes |
| 4. Imbalance | Borderline-SMOTE or cost-sensitive loss | Prevent majority-class bias |
| 5. Feature extraction | RMS, kurtosis, FFT, wavelet per window | Informative model inputs |
| 6. Windowing | 30-step window; 50% overlap | Maximise training samples |
| 7. Splitting | 70/15/15%, temporal + machine-stratified | Prevent leakage; test on unseen machines |

# 5. Experimental Design

LSTM and TCN models are evaluated on fault classification and RUL regression using PRONOSTIA (primary) and C-MAPSS FD001/FD002 (secondary).

## 5.1 Evaluation Metrics

Classification performance is measured by precision, recall, F1 score, and AUC-ROC. Recall is prioritised for the failure-precursor class to minimise missed alarms. RUL regression uses RMSE, MAE, and the NASA PHM Prognostics Scoring Function which asymmetrically penalises late predictions as the domain-relevant metric (Elsherif et al., 2025). RMSE below 20 cycles on C-MAPSS FD001 is the state-of-the-art threshold. Confusion matrices and Wilcoxon signed-rank tests are used for statistical validation.

## 5.2 Cross-Validation Strategy

Standard k-fold cross-validation is unsuitable for time-series as it violates temporal ordering. A walk-forward scheme is used: five chronological folds are trained sequentially on all prior folds. Leave-one-machine-out (LOMO) evaluation additionally tests generalisation to unseen machines. A direct proxy for industrial deployability.

## 5.3 Comparison Criteria

Both models share identical preprocessing pipelines and data splits. Hyperparameters are tuned independently via Bayesian optimisation (50 trials) on the validation fold, using the Adam optimiser with early stopping (max 200 epochs). All runs are replicated five times with different seeds; mean and standard deviation are reported. Ablation studies compare LSTM versus Bidirectional LSTM, and TCN-Attention versus TCN without attention.

Table 3: Experimental Design Summary

| Component | LSTM | TCN |
| --- | --- | --- |
| Datasets | PRONOSTIA (primary); C-MAPSS FD001/FD002 | PRONOSTIA (primary); C-MAPSS FD001/FD002 |
| Tasks | Fault classification + RUL regression | Fault classification + RUL regression |
| Cross-validation | Walk-forward (5 folds) + LOMO | Walk-forward (5 folds) + LOMO |
| Metrics | Precision, Recall, F1, AUC-ROC, RMSE, MAE | Precision, Recall, F1, AUC-ROC, RMSE, MAE |
| Hyperparameter search | Bayesian optimisation (50 trials) | Bayesian optimisation (50 trials) |
| Replication | 5 seeds; mean ± SD | 5 seeds; mean ± SD |
| Ablation | LSTM vs. Bidirectional LSTM | TCN vs. TCN without attention |

# 6. Critical Analysis: Interpretability, Scalability, and Deployment

## 6.1 Interpretability

Interpretability is critical in safety-sensitive manufacturing, where engineers must act on model outputs with confidence. Tree-based ensembles (Random Forest, Gradient Boosting) offer inherent transparency through feature importance scores mappable to engineering variables such as bearing temperature or vibration RMS (Rezk et al., 2024). LSTM and TCN internal representations hidden states, convolutional filters have no direct physical interpretation, embodying the classical deep learning black-box problem.

Post-hoc tools partially close this gap. SHAP (SHapley Additive exPlanations) attributes each model prediction to individual input features. Alotaibi and Aly (2025) show that SHAP-explained transformer-boosting outputs allow operators to trace anomaly alerts to specific sensor channels. LIME fits locally linear surrogates around individual predictions as an alternative. Both methods apply to LSTM and TCN models, though their approximations may not fully reflect true model reasoning. A concern for shutdown-critical decisions. A practical hybrid uses deep learning for detection and RUL estimation, with a tree-based model providing auditable justification for shutdown recommendations.

## 6.2 Scalability

LSTM’s sequential computation scales poorly with fleet size: training time grows super-linearly as more machines are instrumented, demanding dedicated GPU infrastructure for periodic retraining. TCNs benefit from full parallelisation; their fixed receptive field also means memory does not grow with sequence length as LSTM hidden states do. However, TCN dilation factors must be reconfigured as sensor counts grow to maintain adequate temporal coverage. Both architectures support federated learning distributing training across edge devices without centralising raw data which is essential for multi-site deployments with bandwidth or privacy constraints (Alotaibi and Aly, 2025). Model versioning pipelines must include distribution-drift monitoring to detect silent degradation as operating conditions evolve (Sai and Madhavi, 2024).

## 6.3 Deployment

Live deployment requires sub-100ms inference latency for real-time pump and motor anomaly detection. TCN models export cleanly to ONNX and run efficiently on edge accelerators (NVIDIA Jetson, Intel Movidius), whereas LSTM’s stateful computation complicates on-device execution and typically requires INT8 quantisation at the cost of minor accuracy loss (Sai and Madhavi, 2024). SCADA and MES systems were not designed for probabilistic AI outputs; middleware translating predictions into maintenance work orders aligned with OPC-UA communication standards must be custom-built and validated per vendor. Model drift, arising from sensor calibration changes, machine refurbishments, or load shifts, demands a ground-truth feedback loop logging actual failures against predictions. Kullback–Leibler divergence can serve as an unsupervised drift detector; incremental fine-tuning on recent data preserves historical representations and mitigates catastrophic forgetting (Sai and Madhavi, 2024).

# 7. Concluding Remarks

This essay has critically examined AI-based PdM for smart manufacturing, synthesising twelve sources from 2021–2025. Deep learning particularly LSTM, TCN, and hybrid architectures dominates the field, with C-MAPSS and PRONOSTIA as principal benchmarks. Key gaps remain: scarce labelled failure data, poor cross-machine generalisation, and under-addressed edge deployment challenges.

LSTM and TCN were selected and justified for the case study context. LSTM provides a well-validated baseline; TCN offers parallelism and edge-friendliness. The proposed experimental design employs walk-forward cross-validation, LOMO evaluation, and a multi-metric framework spanning both classification and regression. Critical analysis shows that interpretability is a shared limitation, partially addressed by SHAP and LIME; scalability favours TCNs; and deployment demands edge-cloud trade-off decisions, SCADA integration, and drift monitoring. A hybrid TCN–tree-based architecture represents the most pragmatic industrial solution. Future work should pursue domain adaptation for cross-machine transfer and federated learning for privacy-preserving multi-site deployment.

# References

Aboshosha, A., Haggag, A., George, N., Alotaibi, F., Amin, M. and Hamed, A. (2023) IoT-based data-driven predictive maintenance relying on fuzzy system and artificial neural networks. Scientific Reports, 13, 12186. Available at: https://doi.org/10.1038/s41598-023-38887-z [Accessed 31 May 2026].

Abdelhedi, K. and Bouri, A. (2025) A TCN-attention fusion model for fault prediction and remaining useful life estimation of large-scale mining equipment. Scientific Reports. Available at: https://doi.org/10.1038/s41598-026-43145-z [Accessed 31 May 2026].

Alotaibi, N.M. and Aly, M. (2025) SHAP enhanced transformer GWO boosting model for transparent and robust anomaly detection in IIoT environments. Scientific Reports. Available at: https://doi.org/10.1038/s41598-025-25033-0 [Accessed 31 May 2026].

Bampoula, X., Nikolakis, N. and Alexopoulos, K. (2024) Condition monitoring and predictive maintenance of assets in manufacturing using LSTM-autoencoders and transformer encoders. Sensors, 24(10), 3215. Available at: https://doi.org/10.3390/s24103215 [Accessed 31 May 2026].

Bampoula, X., Siaterlis, G., Nikolakis, N. and Alexopoulos, K. (2021) A deep learning model for predictive maintenance in cyber-physical production systems using LSTM autoencoders. Sensors, 21(3), 972. Available at: https://doi.org/10.3390/s21030972 [Accessed 31 May 2026].

Elkateb, S., Métwalli, A., Shendy, A. and Abu-Elanien, A.E.B. (2024) Machine learning and IoT-based predictive maintenance approach for industrial applications. Alexandria Engineering Journal, 88, pp.298–309. Available at: https://doi.org/10.1016/j.aej.2023.12.065 [Accessed 31 May 2026].

Elsherif, S.M., Hafiz, B., Makhlouf, M.A. and Farouk, O. (2025) A deep learning-based prognostic approach for predicting turbofan engine degradation and remaining useful life. Scientific Reports. Available at: https://doi.org/10.1038/s41598-025-09155-z [Accessed 31 May 2026].

Fatahi, A.T.W.K., Kashyzadeh, K.R. and Ghorbani, S. (2024) Advancements in gas turbine fault detection: a machine learning approach based on the temporal convolutional network–autoencoder model. Applied Sciences, 14(11), 4551. Available at: https://doi.org/10.3390/app14114551 [Accessed 31 May 2026].

Pandya, D., Upadhyay, S.H. and Harsha, S.P. (2021) Deep learning-based anomaly-onset aware remaining useful life estimation of bearings. PeerJ Computer Science, 7, e782. Available at: https://doi.org/10.7717/peerj-cs.782 [Accessed 31 May 2026].

Rezk, H., Osheba, I.H.M. and Abdelkader, M. (2024) Optimised predictive maintenance for streaming data in industrial IoT networks using deep reinforcement learning and ensemble techniques. Scientific Reports, 14, 17492. Available at: https://doi.org/10.1038/s41598-025-10268-8 [Accessed 31 May 2026].

Sai, N.L.R. and Madhavi, K.R. (2024) Edge-cloud synergy for AI-enhanced sensor network data: a real-time predictive maintenance framework. Applied Sciences, 14(24), 11866. Available at: https://doi.org/10.3390/app142411866 [Accessed 31 May 2026].

Sharma, K. (2024) Uncertainty-aware deep learning framework for remaining useful life prediction in turbofan engines with learned aleatoric uncertainty. arXiv preprint arXiv:2511.19124. Available at: https://arxiv.org/abs/2511.19124 [Accessed 31 May 2026].